Section 258.APPENDIX C  Soil Closure Objectives

|  |  | RECOMMENDED | SURFACE |
| --- | --- | --- | --- |
| PESTICIDES | CAS No.a | TEST METHODb | (mg/kg) |
| acetochlor | 34256-82-1 | 8151 | 0.9* |
| acifluorfen sodium | 62476-59-9 | 8151 | 2.8 |
| alachlor | 15972-60-8 | 8081 | 1.7* |
| aldicarb | 116-06-3 | 8321 | 1.6* |
| aldrin | 309-00-2 | 8081 | 0.02 |
| atrazine | 1912-24-9 | 8141 | 1.7* |
| bentazon sodium | 50723-80-3 | 8151 | 2.6 |
| bromacil | 314-40-9 | 8321 | 5.2 |
| bromoxynil (o) | 1689-99-2 | 8270 | 6.9 |
| butylate | 2008-41-5 | 8270 | 27 |
| carbofuran | 1563-66-2 | 8270 | 0.6 |
| chlordane | 57-74-9 | 8081 | 29 |
| chlorimuron-ethyl | 90982-32-4 | 8081 | 3.6 |
| chlorpyrifos | 2921-88-2 | 8141 | 50 |
| cyanazine | 21725-46-2 | 8141 | 1.1* |
| 2,4-D | 94-75-7 | 8151 | 1.1 |
| 4,4'-DDD | 72-54-8 | 8081 | 22 |
| 4,4'-DDE | 72-55-9 | 8081 | 64 |
| 4,4'-DDT | 50-29-3 | 8081 | 66 |
| diazinon | 333-41-5 | 8141 | 0.2 |
| dicamba | 1918-00-9 | 8151 | 1.4 |
| dieldrin | 60-57-1 | 8081 | 0.01 |
| dimethoate | 60-51-5 | 8141 | 0.3* |
| dinoseb | 88-85-7 | 8151 | 0.08 |
| disulfoton | 298-04-4 | 8141 | 0.2* |
| endosulfan | 115-29-7 | 8081 | 600 |
| endothall | 145-73-3 | 8270 | 2.4 |
| endrin | 72-20-8 | 8081 | 5 |
| EPTC | 759-94-4 | 8270 | 10 |
| glyphosate | 1071-83-6 | 8321 | 350 |
| HCH-alpha | 319-84-6 | 8081 | 0.01 |
| heptachlor | 76-44-8 | 8081 | 2.3 |
| heptachlor epoxide | 1024-57-3 | 8081 | 3.8 |
| lindane | 58-89-9 | 8081 | 0.07 |
| linuron | 330-55-2 | 8321 | 1.7 |
| malathion | 121-75-5 | 8141 | 41 |
| methoxychlor | 72-43-5 | 8151 | 730 |
| metolachlor | 51218-45-2 | 8151 | 22 |
| metribuzin | 21087-64-9 | 8270 | 2.8 |
| parathion, ethyl | 56-38-2 | 8141 | 77 |
| parathion, methyl | 298-00-0 | 8141 | 2.6 |
| pendimethalin | 40487-42-1 | 8091 | 900 |
| permethrin | 52645-53-1 | 8081 | 3300 |
| phorate | 298-02-2 | 8141 | 1.1* |
| simazine | 122-34-9 | 8141 | 1.7* |
| 2,4,5-TP | 93-72-1 | 8270 | 65 |
| terbufos | 13071-79-9 | 8141 | 0.4* |
| toxaphene | 8001-35-2 | 8081 | 72 |
| trifluralin | 1582-09-8 | 8091 | 100 |

| NUTRIENTS | SURFACE |  |  |
| --- | --- | --- | --- |
| Ammonia (as N) plus Nitrate (as N) | 100 mg/kg |  |  |
| Phosphorous (Bray P1) | 150 mg/kg |  |  |

| Notes: | a Chemical Abstract Service (CAS) |
| --- | --- |
|  | b USEPA Test Methods (SW-846) |
|  | * Application Rate Equivalents (AREs) are based on USEPA-approved pesticide label rates for the specific pesticide active ingredient and conservative assumptions about soil properties. AREs only apply to the upper three inches of soil and are used as the Soil Cleanup Objective (SCO) if the ARE is greater than the SCO value calculated using the Equation in 8 Ill. Adm. Code 259.350(a). |

The application rate equivalents can be determined using the equation below:

| ARE (mg/kg) | = | Application Rate (1b/acre) x (1 acre-foot / 43,560 ft3) x (1 ft3 / 110 1b) x (1 / 0.25 ft) x (1.0 x 106 mg/kg) |
| --- | --- | --- |

Where:

ARE = application rate equivalent (mg/kg) for coarse-textured, low organic matter content soils

Applicationrate = current label application rate (1b/acre)

(Source:  Old Section 258.APPENDIX C renumbered to Section 258.APPENDIX D, New Section 258.APPENDIX C added at 26 Ill. Reg. 17155, effective November 18, 2002)