Section 320.370  Permanent Storage Area Survey Record

a)	Each custodian of a radiological training source set shall make a sketch of the permanent storage area and indicate the location of the radiological training source set in their possession with the proximity of other areas of the building.

b)	Using a CD V-700 low range survey meter with the shield closed, each custodian shall survey outside of the access doors and any exterior walls of the room where the source set is stored.  The outside radiation survey levels shall be recorded on the sketch.

c)	Any reading in excess of 2 mr/hr shall be reported to the IESDA Radiological Protection Officer.

d)	Area surveys shall be made on each January and July.

e)	New surveys are required if the permanent storage area is altered or if the number of radiological training source sets has changed.

f)	The IESDA Radiological Protection Officer shall inspect each storage area annually to determine if the requirements of 10 CFR 20 and this part are being complied with.