Section 315.TABLE B   MPE for Skin Exposure

Ultraviolet

| Wavelength (μm) | Exposure Duration, ts | MPE |  |
| --- | --- | --- | --- |
|  |  | (J cm-2) | (W cm-2) |
| 0.180 to 0.302 | 10-9 to 3 x 104 | 3 x 10-3 |  |
| 0.303 | 10-9 to 3 x 104 | 4 x 10-3 |  |
| 0.304 | 10-9 to 3 x 104 | 6 x 10-3 |  |
| 0.305 | 10-9 to 3 x 104 | 1.0 x 10-2 |  |
| 0.306 | 10-9 to 3 x 104 | 1.6 x 10-2 |  |
| 0.307 | 10-9 to 3 x 104 | 2.5 x 10-2 |  |
| 0.308 | 10-9 to 3 x 104 | 4.0 x 10-2 |  |
| 0.309 | 10-9 to 3 x 104 | 6.3 x 10-2 |  |
| 0.310 | 10-9 to 3 x 104 | 1.0 x 10-1 |  |
| 0.311 | 10-9 to 3 x 104 | 1.6 x 10-1 |  |
| 0.312 | 10-9 to 3 x 104 | 2.5 x 10-1 |  |
| 0.313 | 10-9 to 3 x 104 | 4.0 x 10-1 |  |
| 0.314 | 10-9 to 3 x 104 | 6.3 x 10-1 |  |
| 0.315 to 0.400 | 10-9 to 10 | 0.56 t¼ |  |
| 0.315 to 0.400 | 10 x 103 | 1 |  |
| 0.315 to 0.400 | 103 to 3 x 104 |  | 1 x 10-3 |

| NOTES: | 1. | To calculate MPE, use the J cm-2 value shown or 0.56 t¼, whichever is lower. |
| --- | --- | --- |
|  | 2. | 3.5 mm limiting aperture (see Section 315.Table D). |

Visible and Near Infrared

| Wavelength (μm) | Exposure Duration, ts | MPE |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | (J cm-2) |  | (W cm-2) |  |
| 0.400 to 1.400 | 10-9 to 10-7 | 2CA x 10-2 |  |  |
|  | 10-7 to 10 | 1.1CA t¼ |  |  |
|  | 10 to 3 x 104 |  | 0.2CA |  |

| NOTE: | 3.5 mm limiting aperture (see Section 315.Table D). |
| --- | --- |

Far Infrared

| Wavelength (μm) | Exposure Duration, ts | MPE |  |
| --- | --- | --- | --- |
|  |  | (J cm-2) | (W cm-2) |
| 1.400 to 103 | 10-9 to 10-7 | 10-2 |  |
|  | 10-7 to 10 | 0.56 t¼ |  |
|  | >10 |  | 0.1 |

| NOTE: | See Section 315.Table D for limiting apertures. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| GENERAL NOTE: |  | 1. | For purposes of this Table, the following abbreviations are used: |  |  |  |
|  |  |  |  | μm | = | micrometers |
|  |  |  |  | ts | = | time in seconds |
|  |  |  |  | J | = | joules |
|  |  |  |  | W | = | watts |
|  |  |  |  | cm | = | centimeters |
|  |  |  |  | mm | = | millimeter |