Section 315.TABLE E   Measurement Apertures for Classification

| Spectral Region (μm) |  |  |  |  |  | Duration (s) |  | Aperture Diameter (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.180 to 0.302 |  |  |  |  |  | 10-9 to 0.25 |  | 1.0 |
|  |  |  |  |  |  | 0.25 to 3 x 104 |  | 3.5 |
| 0.302 to 2.8 |  |  |  |  |  | 10-9 to 3 x 104 |  | 50.0 |
| 2.8 to 102 |  |  |  |  |  | 10-9 to 0.3 |  | 1.0 |
|  |  |  |  |  |  | 0.3 to 10 |  | 1.5 t3/8 |
|  |  |  |  |  |  | 10 to 3 x 104 |  | 3.5 |
| 102 to 103 |  |  |  |  |  | 10-9 to 3 x 104 |  | 11.0 |
| NOTES: | 1. | These apertures are used for the measurement of optical power or energy for purposes of laser classification |  |  |  |  |  |  |
|  | 2. | When the laser output is intended to be viewed with optics (excluding ordinary eyeglasses) or the laser safety officer determines that there is reasonable probability of accidental viewing with optics, a 50 mm aperture is used if the following conditions are met. |  |  |  |  |  |  |
|  |  | A) | Viewing with optics presents a more severe hazard than unaided viewing. |  |  |  |  |  |
|  |  | B) | The viewing time is sufficient to constitute a hazard. |  |  |  |  |  |
|  | 3. | Under normal conditions these exposure durations would not be used for classification (see ANSI Z136.1 (Table 9)). |  |  |  |  |  |  |
|  | 4. | For purposes of this Section 315.Table E, the following abbreviations or symbols are used: |  |  |  |  |  |  |
|  |  |  |  | μm | = |  | micrometers |  |
|  |  |  |  | s | = |  | seconds |  |
|  |  |  |  | mm | = |  | millimeters |  |
|  |  |  |  | ts | = |  | time in seconds |  |