Section 201.635  PBR Boiler Recordkeeping Requirements

The owner or operator of the PBR boiler must maintain records containing the following information, in addition to the records required by Section 201.530:

a)	The maximum design heat input capacity of the boiler, in MMBtu/hr, with supporting documentation;

b)	An inspection, maintenance, and repair log with dates and the nature of those activities for the boiler;

c)	The quantity of each fuel used per month and per year;

d)	The hours of operation, in hours/month and hours/year;

e)	Emissions of PM, PM10, PM2.5, NOx, CO, and VOM, in tons/month and tons/year, with supporting calculations; and

f)	SO2 emissions, in tons/month and tons/year, with supporting calculations if the boiler has the capability to burn refinery fuel gas, butane, or propane.

(Source:  Added at 41 Ill. Reg. 4140, effective March 24, 2017)