Section 204.280  Best Available Control Technology (BACT)

"Best Available Control Technology" means an emissions limitation (including a visible emission standard) based on the maximum degree of reduction for each regulated NSR pollutant that would be emitted from any proposed major stationary source or major modification that the Agency, on a case-by-case basis, taking into account energy, environmental, and economic impacts and other costs, determines is achievable for such source or modification through application of production processes or available methods, systems, and techniques, including fuel cleaning or treatment or innovative fuel combustion techniques for control of such pollutant.  In no event shall application of BACT result in emissions of any pollutant that would exceed the emissions allowed by any applicable standard under 40 CFR 60, 61, 62 and 63 (incorporated by reference in Section 204.100).  If the Agency determines that technological or economic limitations on the application of measurement methodology to a particular emissions unit would make the imposition of an emissions standard infeasible, a design, equipment, work practice, operational standard, or combination thereof, may be prescribed instead to satisfy the requirement for the application of BACT.  Such standard shall, to the degree possible, set forth the emissions reduction achievable by implementation of such design, equipment, work practice or operation, and shall provide for compliance by means that achieve equivalent results.