Section 204.510  Major Stationary Source

a)	"Major stationary source" means:

1)	Any of the following stationary sources of air pollutants that emits, or has the potential to emit, 100 tpy or more of any regulated NSR pollutant:

A)	Fossil fuel-fired steam electric plants of more than 250 million Btu per hour heat input;

B)	Coal cleaning plants (with thermal dryers);

C)	Kraft pulp mills;

D)	Portland cement plants;

E)	Primary zinc smelters;

F)	Iron and steel mill plants;

G)	Primary aluminum ore reduction plants (with thermal dryers);

H)	Primary copper smelters;

I)	Municipal incinerators capable of charging more than 50 tons of refuse per day;

J)	Hydrofluoric, sulfuric, and nitric acid plants;

K)	Petroleum refineries;

L)	Lime plants;

M)	Phosphate rock processing plants;

N)	Coke oven batteries;

O)	Sulfur recovery plants;

P)	Carbon black plants (furnace process);

Q)	Primary lead smelters;

R)	Fuel conversion plants;

S)	Sintering plants;

T)	Secondary metal production plants;

U)	Chemical process plants (which does not include ethanol production facilities that produce ethanol by natural fermentation included in NAICS Codes 325193 or 312140);

V)	Fossil-fuel boilers (or combinations thereof) totaling more than 250 million Btu per hour heat input;

W)	Petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels;

X)	Taconite ore processing plants;

Y)	Glass fiber processing plants; and

Z)	Charcoal production plants;

2)	Notwithstanding the stationary source size specified in subsection (a)(1), any stationary source that emits, or has the potential to emit, 250 tpy or more of a regulated NSR pollutant (except GHGs as defined in Section 204.430); or

3)	Any physical change that would occur at a stationary source not otherwise qualifying under this Section as a major stationary source, if the changes would constitute a major stationary source.

b)	A major source that is major for VOM or NOX shall be considered major for ozone.

c)	The fugitive emissions of a stationary source shall not be included in determining for any of the purposes of this Part whether it is a major stationary source, unless the source belongs to one of the following categories of stationary sources:

1)	Coal cleaning plants (with thermal dryers);

2)	Kraft pulp mills;

3)	Portland cement plants;

4)	Primary zinc smelters;

5)	Iron and steel mills;

6)	Primary aluminum ore reduction plants;

7)	Primary copper smelters;

8)	Municipal incinerators capable of charging more than 50 tons of refuse per day;

9)	Hydrofluoric, sulfuric, or nitric acid plants;

10)	Petroleum refineries;

11)	Lime plants;

12)	Phosphate rock processing plants;

13)	Coke oven batteries;

14)	Sulfur recovery plants;

15)	Carbon black plants (furnace process);

16)	Primary lead smelters;

17)	Fuel conversion plants;

18)	Sintering plants;

19)	Secondary metal production plants;

20)	Chemical process plants.  Chemical processing plant shall not include ethanol production facilities that produce ethanol by natural fermentation included in NAICS Codes 325193 or 312140;

21)	Fossil-fuel boilers (or combination thereof) totaling more than 250 million Btu per hour heat input;

22)	Petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels;

23)	Taconite ore processing plants;

24)	Glass fiber processing plants;

25)	Charcoal production plants;

26)	Fossil fuel-fired steam electric plants of more than 250 million Btu per hour heat input; and

27)	Any other stationary source category that, as of August 7, 1980, is being regulated under section 111 or 112 of the CAA.