Section 212.205  Coal-fired Industrial Boilers For Which Construction or Modification Commenced Prior to April 14, 1972, Equipped with Flue Gas Desulfurization Systems

Notwithstanding Sections 212.201 through 212.204 of this Subpart, no person shall cause or allow the emission of particulate matter into the atmosphere from coal-fired industrial boilers equipped with flue gas desulfurization systems for which construction or modification commenced prior to April 14, 1972, to exceed 0.39 kg of particulate matter per MW-hr of actual heat input in any one-hour period (0.25 lbs/mmbtu).  Nothing in this rule shall be construed to prevent compliance with applicable regulations  promulgated by the USEPA under Section 111 of the CAA as amended. The provisions of Section 111 of the Clean Air Act relating to standards of performance for new stationary sources...are applicable in this State and are enforceable under the Act [415 ILCS 5/9.1(b)].

(Source:  Amended at 20 Ill. Reg. 7605, effective May 22, 1996)