Section 214.161  Liquid Fuel Burned Exclusively

a)	Prior to January 1, 2017, no person shall cause or allow the emission of sulfur dioxide into the atmosphere in any one hour period from any existing fuel combustion emission source, burning liquid fuel exclusively, to exceed the following:

1)	1.55 kg of sulfur dioxide per MW-hr of actual heat input when residual fuel oil is burned (1.0 lbs/mmBtu); and

2)	0.46 kg of sulfur dioxide per MW-hr of actual heat input when distillate fuel oil is burned (0.3 lbs/mmBtu).

b)	Except as provided in subsections (c) and (d), on and after January 1, 2017, the owner or operator of an existing fuel combustion emission source, burning liquid fuel exclusively, must comply with the following:

1)	The sulfur content of all residual fuel oil used by the fuel combustion emission source must not exceed 1000 ppm;

2)	The sulfur content of all distillate fuel oil used by the fuel combustion emission source must not exceed 15 ppm; and

3)	The owner or operator must:

A)	Maintain records demonstrating that the fuel oil used by the fuel combustion emission source complies with the requirements in subsections (b)(1) and (b)(2), such as records from the fuel supplier indicating the sulfur content of the fuel oil;

B)	Retain the records for at least 5 years, and provide copies of the records to the Agency within 30 days after receipt of a request by the Agency; and

C)	Notify the Agency within 30 days after discovery of deviations from any of the requirements in this subsection (b).  At minimum, and in addition to any permitting obligations, the notification must include a description of the deviations, a discussion of the possible cause of the deviations, any corrective actions taken, and any preventative measures taken.

c)	The sulfur content limitation for distillate fuel oil in subsection (b)(2) does not apply to existing electric generating units at Midwest Generation's Joliet station (located at or near 1800 Channahon Road, Joliet IL), Powerton station (located at or near 13082 E. Manito Road, Pekin IL), Waukegan station (located at or near 401 E. Greenwood Avenue, Waukegan IL), and Will County station (located at or near 529 E. 135th, Romeoville IL).  The owner or operator of such electric generating units must instead comply with the following:

1)	From January 1, 2016 through December 31, 2018, the sulfur content of all distillate fuel oil purchased for use by such electric generating units must not exceed 15 ppm;

2)	From January 1, 2017 through December 31, 2018, the sulfur content of all distillate fuel oil used by such electric generating units must not exceed 500 ppm;

3)	On and after January 1, 2019, the sulfur content of all distillate fuel oil used by such electric generating units must not exceed 15 ppm;

4)	The owner or operator must:

A)	Maintain records demonstrating that the distillate fuel oil purchased from January 1, 2016 through December 31, 2018 for use by the electric generating units complies with the requirements in subsection (c)(1), such as records from the fuel supplier indicating the sulfur content of the fuel oil, and maintain records indicating the date of purchase of the fuel oil;

B)	Maintain records demonstrating that the distillate fuel oil used from January 1, 2017 through December 31, 2018, by the electric generating units, complies with the requirements in subsection (c)(2), such as records from the fuel supplier indicating the sulfur content of the fuel oil;

C)	On and after January 1, 2019, maintain records demonstrating that the distillate fuel oil used by the electric generating units complies with the requirements in subsection (c)(3), such as records from the fuel supplier indicating the sulfur content of the fuel oil;

D)	Retain all records required by this subsection (c) for at least 5 years, and provide copies of the records to the Agency within 30 days after receipt of a request by the Agency; and

E)	Notify the Agency within 30 days after discovery of deviations from any of the requirements in this subsection (c).  At minimum, and in addition to any permitting obligations, the notification must include a description of the deviations, a discussion of the possible cause of the deviations, any corrective actions taken, and any preventative measures taken.

d)	The sulfur content limitation for distillate fuel oil in subsection (b)(2) does not apply to existing fuel combustion emission sources at Caterpillar's Montgomery facility (located at or near 325 South Route 31, Montgomery IL).  The owner or operator of the fuel combustion emission sources must instead comply with the following:

1)	On and after January 1, 2016:

A)	The sulfur content of all distillate fuel oil purchased for use by the fuel combustion emission sources must not exceed 15 ppm; and

B)	The sulfur content of all distillate fuel oil used by the fuel combustion emission sources must not exceed 500 ppm;

2)	The owner or operator must:

A)	Maintain records demonstrating that the distillate fuel oil purchased on and after January 1, 2016 for use by the fuel combustion emission sources complies with the requirements in subsection (d)(1)(A), such as records from the fuel supplier indicating the sulfur content of the fuel oil, and maintain records indicating the date of purchase of the fuel oil;

B)	Maintain records demonstrating that the distillate fuel oil used on and after January 1, 2016 by the fuel combustion emission sources complies with the requirements in subsection (d)(1)(B), such as records from the fuel supplier indicating the sulfur content of the fuel oil;

C)	Retain all records required by this subsection (d) for at least 5 years, and provide copies of the records to the Agency within 30 days after receipt of a request by the Agency; and

D)	Notify the Agency within 30 days after discovery of deviations from any of the requirements in this subsection (d).  At minimum, and in addition to any permitting obligations, the notification must include a description of the deviations, a discussion of the possible cause of the deviations, any corrective actions taken, and any preventative measures taken.

(Source:  Amended at 39 Ill. Reg. 16174, effective December 7, 2015)