Section 214.303  Use of Sulfuric Acid

With the exception of fuel combustion emission sources and acid manufacturing, no person using sulfuric acid shall cause or allow the emission of sulfuric acid and/or sulfur trioxide from all other similar emission sources at a plant or premises to exceed:

a)	45.4 grams in any one hour period for sulfuric acid usage less than 1180 Mg/yr (100 percent acid basis) (0.10 lbs/hr up to 1300 T/yr);

b)	250 grams per metric ton of acid used for sulfuric acid usage greater than or equal to 1180 Mg/yr (100 percent acid basis) (0.50 lbs/T over 1300 T/yr).

(Source:  Amended at 7 Ill. Reg. 4219, effective March 28, 1983)