Section 215.462  Green Tire Spraying Operations

The owner or operator of a green tire spraying operation at a pneumatic rubber tire manufacturing facility shall:

a)	Install and operate:

1)	A capture system with a minimum capture efficiency of 90 percent by weight of volatile organic material; and

2)	A control device that meets the requirements of one of the following:

A)	A carbon adsorption system designed and operated in a manner such that there is at least 90 percent removal of volatile organic material by weight from the bases ducted to the control device;

B)	An afterburning system that oxidizes at least 90 percent of the captured non-methane volatile organic material (measured as total combustible carbon) to carbon dioxide and water; or

C)	An alternative volatile organic material emission reduction system demonstrated to have at least a 90 percent overall reduction efficiency and approved by the Agency.

b)	Substitute for the normal solvent-based mold release compound water-based sprays containing:

1)	No more than five percent by volume of volatile organic material as applied for the inside of tires;

2)	No more than ten percent by volume of volatile organic material as applied for the outside of tires.

(Source:  Added at 7 Ill. Reg. 1244, effective January 21, 1983)