Section 215.488  Monitors for Air Pollution Control Equipment

a)	At a minimum, continuous monitors for the following parameters shall be installed on air pollution control equipment subject to this Subpart:

1)	Destruction device combustion temperature;

2)	Temperature rise across a catalytic afterburner bed;

3)	Breakthrough of volatile organic material on a carbon adsorption unit;

4)	Outlet gas temperature of a refrigerated condenser;

5)	Temperature of a non-refrigerated condenser coolant supply system.

b)	Each monitor shall be equipped with a recording device.

c)	Each monitor shall be calibrated quarterly.

d)	Each monitor shall operate at all times while the associated control equipment is operating.

(Source:  Amended at 15 Ill. Reg. 8018, effective May 14, 1991)