Section 215.510  Coke By-Product Recovery Plants

The owner or operator of a coke by-product recovery plant shall reduce the uncontrolled emissions of volatile organic materials by at least 85 percent from the following sources, as defined:

a)	Tar decanter, which is a rectangular vessel used to separate tar and flushing liquor by means of gravity;

b)	Light oil sump, which receives wastewater from process equipment from the light oil recovery portion of a coke by-product recovery plant;

c)	Light oil condensor/separator, which is a device used to condense or separate light oil from which the non-condensable constituents are vented; and

d)	Tar condensate sump, which receives water condensate streams from the tar recovery process equipment.

(Source:  Added at 9 Ill. Reg. 13960, effective August 28, 1985)