Section 215.521  Definitions

In addition to the definitions of 35 Ill. Adm. Code 211, the following definitions apply to this Subpart:

"Air Oxidation Process":  any unit process including ammoxidation and oxychlorination which uses air or a combination of air and oxygen as an oxidant in combination with one or more organic reactants to produce one or more organic compounds.

"Cost Effectiveness":  the annual expense for cost of control of a given process stream divided by the reduction in emissions of organic material of that stream.

"Flow (F)":  Vent stream flowrate (scm/min) at a standard temperature of 20C.

"Full Operating Flowrate":  Maximum operating capacity of the facility.

"Hourly Emissions (E)":  Hourly emissions reported in kg/hr measured at full operating flowrate.

"Net Heating Value (H)":  Vent stream net heating value (MJ/scm), where the net enthalpy per mole of offgas is based on combustion at 25º C and 760 mm Hg, but the standard temperature for determining the volume corresponding to one mole is 20º C, as in the definition of "Flow."

"Process vent Stream":  an emission stream resulting from an air oxidation process.

"Total Resource Effectiveness Index (TRE)":  Cost effectiveness in dollars per megagram of controlling any gaseous stream vented to the atmosphere from an air oxidation process divided by $1600/Mg, using the criteria and methods set forth in this Subpart and Appendices E and F.

(Source:  Added at 11 Ill. Reg. 20829, effective December 14, 1987)