Section 215.926  Control Requirements

a)	Every owner or operator of an emission source of volatile organic material shall operate in compliance with RACT, which for emission sources subject to this Subpart shall be:

1)	Emission capture and control techniques which achieve an overall reduction in uncontrolled volatile organic material emissions of at least 81%; or

2)	For coating lines, volatile organic material emissions not to exceed 0.42 kg/l (3.5 lb/gal) of coating materials as applied, excluding water and any compounds which are specifically exempted from the definition of volatile organic material, on a daily basis.  Owners and operators complying with this subsection are not required to comply with Section 215.301; or

3)	An adjusted RACT emissions limitation obtained pursuant to Subpart I.

b)	Owners and operators of emission sources subject to this Subpart shall comply with its requirements by April 1, 1989.

(Source:  Added at 12 Ill. Reg. 7311, effective April 8, 1988)