Section 217.396  Recordkeeping and Reporting

a)	Recordkeeping.  The owner or operator of any unit included in an emissions averaging plan (e.g., affected units, nonsubject units, units that could be exempt under Section 217.386(c), and low usage units) or an affected unit that is not exempt under Section 217.386(c) and is not subject to the low usage exemption of Section 217.388(a)(3) must maintain records that demonstrate compliance with the requirements of this Subpart Q, which include, but are not limited to:

1)	Identification, type (e.g., lean-burn, gas-fired), and location of each unit.

2)	Calendar date of the record.

3)	Before July 1, 2025, the number of hours the unit operated on a monthly basis and during each ozone season.  On and after July 1, 2025, daily operating hours.

4)	Type and quantity of the fuel used on a daily basis.

5)	On and after July 1, 2025, total mass emissions on a daily basis and on a 30-day rolling average basis.

6)	The results of all monitoring performed on the unit and reported deviations.

7)	The results of all tests performed on the unit.

8)	The plan for performing inspection and maintenance of the units, air pollution control equipment, and the applicable monitoring device under Section 217.388(a)(4).

9)	A log of inspections and maintenance performed on the unit's air emissions, monitoring device, and air pollution control device.  These records must include, at a minimum, date, load levels and any manual adjustments, along with the reason for the adjustment (e.g., air to fuel ratio, timing or other settings).

10)	Before July 1, 2025, if complying with the emissions averaging plan provisions of Sections 217.388(a)(2) and 217.390, copies of the calculations used to demonstrate compliance with the ozone season and annual control period limits, noncompliance reports for the ozone season, and ozone and annual control period compliance reports submitted to the Agency.

11)	Identification of time periods for which operating conditions and pollutant data were not obtained by either the CEMS or alternate monitoring procedures, including the reasons for not obtaining sufficient data and a description of corrective actions taken.

12)	Any NOx allowance reconciliation reports submitted under Section 217.392(d)(3).

13)	If the engine or turbine is used as an emergency or standby unit, records documenting the annual hours of operation of these units in non-emergency situations.

b)	The owner or operator of an affected unit or unit included in an emissions averaging plan must maintain the records required by subsection (a) or (e) of this Section, as applicable, for a period of five years at the source at which the unit is located.  The records must be made available to the Agency and USEPA upon request.

c)	Reporting Requirements

1)	The owner or operator must notify the Agency in writing 30 days and five days prior to testing, under Section 217.394(a) and (c) and:

A)	If, after the 30-days' notice for an initially scheduled test is sent, there is a delay (e.g., due to operational problems) in conducting the performance test as scheduled, the owner or operator of the unit must notify the Agency as soon as possible of the delay in the original test date, either by providing at least seven days prior notice of the rescheduled date of the performance test or by arranging a new test date with the Agency by mutual agreement;

B)	Provide a testing protocol to the Agency 60 days prior to testing; and

C)	Not later than 30 days after the completion of the test, submit the results of the test to the Agency.

D)	Notwithstanding subsections (c)(1)(A) through (C), 40 CFR Part 75 affected sources may provide testing notification and protocol submittal in accordance with 40 CFR 75.61(a)(1) and (5) and report in accordance with 40 CFR 75.60(b)(7) requirements.

2)	Under the requirements for monitoring in Section 217.394(e), the owner or operator of the unit must report to the Agency any monitored exceedances of the applicable NOx concentration from Section 217.388(a)(1) or (a)(2) within 30 days after performing the monitoring.

3)	Within 90 days after permanently shutting down an affected unit or a unit included in an emissions averaging plan, the owner or operator of the unit must withdraw or amend the applicable permit to reflect that the unit is no longer in service.

4)	Until July 1, 2025, if demonstrating compliance through an emissions averaging plan:

A)	By October 31 following the applicable ozone season, the owner or operator must notify the Agency if he or she cannot demonstrate compliance for that ozone season; and

B)	By January 31 following the applicable calendar year, the owner or operator must submit to the Agency a report that demonstrates the following:

i)	For all units that are part of the emissions averaging plan, the total mass of allowable NOx emissions for the ozone season and for the annual control period;

ii)	The total mass of actual NOx emissions for the ozone season and annual control period for each unit included in the averaging plan;

iii)	The calculations that demonstrate that the total mass of actual NOx emissions are less than the total mass of allowable NOx emissions using equations in Sections 217.390(h) and (i); and

iv)	The information required to determine the total mass of actual NOx emissions and the calculations performed in subsection (c)(4)(B)(iii) of this Section.

5)	On and after July 1, 2025, if demonstrating compliance through an emissions averaging plan, by May 1 following the previous calendar year, the owner or operator must submit to the Agency a report that includes the following:

A)	For all units that are part of the emissions averaging plan, the total mass of allowable NOx emissions on a 30-day rolling average basis.

B)	The total mass of actual NOx emissions on a 30-day rolling average basis for each unit included in the averaging plan.

C)	The calculations that demonstrate that the total mass of actual NOx emissions is less than the total mass of allowable NOx emissions using equations in Sections 217.390(j) and (k).

D)	The daily information required to determine the total mass of actual NOx emissions on a 30-day rolling average basis.

6)	If operating a CEMS, the owner or operator must submit an excess emissions and monitoring systems performance report in accordance with the requirements of 40 CFR 60.7(c) and 60.13 or 40 CFR 75, incorporated by reference in Section 217.104, or an alternate procedure approved by the Agency or USEPA and included in a federally enforceable permit.

7)	If using NOx allowances to comply with the requirements of Section 217.388, reconciliation reports as required by Section 217.392(d)(3).

d)	On and after July 1, 2025, the owner or operator of an emission unit subject to Subpart Q must submit an annual compliance certification report that demonstrates compliance with the applicable requirements to the Agency for the preceding calendar year by May 1 of the following year.  The owner or operator may submit the annual compliance certification report to the Agency along with the Annual Emissions Report required under 35 Ill. Adm. Code 254 or the compliance certification required under 415 ILCS 5/39.5(7)(p)(v).  The compliance report must include the following:

1)	Identification, type (e.g., lean-burn, gas-fired), and location of the emission unit.

2)	Methods used for determining compliance, including an emissions averaging plan, if applicable, a description of test methods, monitoring, recordkeeping, and reporting requirements.

3)	A certification of compliance with the applicable emissions concentration or identification of the periods of noncompliance with a quantification of the excess emissions concentration and the excess emissions.

4)	For each calendar month, the highest 30-day rolling average emission rate.  The emissions data must be reported in the measurement units of the applicable emissions concentration.

5)	The emission unit's daily and total operating hours, capacity utilization, and the percent operation of any CEMS during the hours the emission unit was operating.

6)	A certification of compliance with all applicable requirements except those identified signed by a responsible official that contains the following: "I certify, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete."

e)	The owner or operator of an affected unit that is complying with the low usage provisions of Section 217.388(a)(3) must:

1)	Before July 1, 2025, for each unit complying with Section 217.388(a)(3)(A), maintain a record of the NOx emissions for each calendar year;

2)	For each unit complying with Section 217.388(a)(3)(B), maintain a record of bhp or MW-hours operated each calendar year; and

3)	Before July 1, 2025, for each unit utilizing NOx allowances for compliance under Section 217.392(d)(3), maintain and submit any NOx allowance reconciliation reports.

f)	Instead of complying with the requirements of subsection (a) of this Section, subsection (b) of this Section, subsections (c)(1) through (c)(5) of this Section, and subsection (e) of this Section, an owner or operator of an affected unit complying with the requirements of Section 217.388(a)(1) and operating a CEMS on that unit may meet the applicable testing, monitoring, reporting and recordkeeping requirements for that CEMS of 40 CFR 75, as incorporated by reference in Section 217.104.

(Source:  Amended at 49 Ill. Reg. 6355, effective April 23, 2025)