Section 219.110  Vapor Pressure of Organic Material or Solvent

a)	If the organic material or solvent consists of only a single compound, the vapor pressure must be determined by ASTM Method D2879-86 (incorporated by reference in Section 219.112 of this Part) or the vapor pressure may be obtained from a publication such as:  Boublik, T., V. Fried and E. Hala, "The Vapor Pressure of Pure Substances," Elsevier Scientific Publishing Co., New York (1973); Perry's Chemical Engineer's Handbook, McGraw-Hill Book Company (1984); CRC Handbook of Chemistry and Physics, Chemical Rubber Publishing Company (1986-87); and Lange's Handbook of Chemistry, John A. Dean, editor, McGraw-Hill Book Company (1985).

b)	Except as provided in subsection (d), if the organic material or solvent is in a mixture made up of both organic material compounds and compounds which are not organic material, the vapor pressure must be determined by the following equation:

where:

| Pom | = | Total vapor pressure of the portion of the mixture which is composed of organic material; |
| --- | --- | --- |
| n | = | Number of organic material components in the mixture; |
| i | = | Subscript denoting an individual component; |
| Pi | = | Vapor pressure of an organic material component determined in accordance with subsection (a); |
| Xi | = | Mole fraction of the organic material component of the total organic mixture. |

c)	If the organic material or solvent is in a mixture made up only of organic material compounds, the vapor pressure must be determined by ASTM Method D2879-86 (incorporated by reference in Section 219.112) or by the above equation.

d)	For hand-wipe cleaning solvents used at aerospace facilities subject to Section 219.219(g)(2), the composite vapor pressure of a cleaning solvent consisting of multiple components must be determined by the following equation:

PPc=i=1nWiMW1    ×  VPiWwMWw  +  j=1nWjMWj  +  i=1nWiMWi

where:

| PPc | = | Composite vapor pressure of the cleaning solvent in mmHg at 20 °C; |
| --- | --- | --- |
| n | = | Number of components in the cleaning solvent; |
| i | = | Subscript denoting an individual VOM-containing component; |
| j | = | Subscript denoting an individual non-VOM component; |
| Wi | = | Weight of a VOM-containing component in grams; |
| Wj | = | Weight of a non-VOM component in grams; |
| Ww | = | Weight of water in grams; |
| MWi | = | Molecular weight a VOM-containing component in grams per gram-mole; |
| MWj | = | Molecular weight of a non-VOM component in grams per gram-mole |
| MWw | = | Molecular weight of water in grams per gram-mole; |
| VPi | = | Vapor pressure of a VOM-containing component in mmHg at 20 °C. |

(Source:  Amended at 45 Ill. Reg. 3553, effective March 4, 2021)