Section 219.462  Green Tire Spraying Operations

The owner or operator of a green tire spraying operation at a pneumatic rubber tire manufacturing source shall:

a)	Install and operate:

1)	A capture system with a minimum capture efficiency of 90 percent by weight of VOM; and

2)	A control device that meets the requirements of one of the following:

A)	A carbon adsorption system designed and operated in a manner such that there is at least 90 percent removal of VOM by weight from the gases ducted to the control device;

B)	An afterburning system that oxidizes at least 90 percent of the captured nonmethane VOM (measured as total combustible carbon) to carbon dioxide and water; or

C)	An alternative VOM emission reduction system demonstrated to have at least a 90 percent overall reduction efficiency approved by the Agency and approved by the USEPA as a SIP revision.

b)	Substitute for the normal solvent-based mold release compound water-based sprays containing:

1)	No more than five percent by volume of VOM as applied for the inside of tires;

2)	No more than ten percent by volume of VOM as applied for the outside of tires.

(Source:  Amended at 17 Ill. Reg. 16918, effective September 27, 1993)