Section 219.488  Monitoring for Air Pollution Control Equipment

a)	At a minimum, continuous monitors for the following parameters shall be installed on air pollution control equipment used to control sources subject to this Subpart:

1)	Destruction device combustion temperature.

2)	Temperature rise across a catalytic afterburner bed.

3)	VOM concentration on a carbon adsorption unit to determine breakthrough.

4)	Outlet gas temperature of a refrigerated condenser.

5)	Temperature of a non-refrigerated condenser coolant supply system.

b)	Each monitor shall be equipped with a recording device.

c)	Each monitor shall be calibrated quarterly.

d)	Each monitor shall operate at all times while the associated control equipment is operating.