Section 241.APPENDIX A   Emission Standards for Clean Fuel Vehicles

Section 241.TABLE A   Low Emission Vehicle (LEV) Standards for Light-Duty Clean Fuel Vehicles (g/mi)

| LIGHT-DUTY VEHICLE WEIGHT | POLLUTANT: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NMOG | CO | NOX | HCHO | PM1 |
| ALL LDV, LDT ≤ 6000 GVWR ≤ 3750 LVW 50,000 MILES 100,000 MILES | 0.075 0.090 | 3.4 4.2 | 0.2 0.3 | 0.015 0.018 | --- 0.80 |
| LDT ≤ 6000 GVWR > 3750 LVW ≤ 5750 LVW 50,000 MILES 100,000 MILES | 0.100 0.130 | 4.4 5.5 | 0.4 0.5 | 0.018 0.023 | --- 0.08 |
| LDT >6000 GVWR ≤ 3750 ALVW 50,000 MILES 100,000 MILES | 0.125 0.180 | 3.4 5.0 | 0.42 0.6 | 0.015 0.022 | --- 0.10 |
| LDT >6000 GVWR >3750 ALVW ≤ 5750 ALVW 50,000 MILES 120,000 MILES | 0.160 0.230 | 4.4 6.4 | 0.72 1.0 | 0.018 0.027 | --- 0.10 |
| LDT >6000 GVWR >5750 ALVW ≤ 8500 ALVW3 50,000 MILES 100,000 MILES | 0.195 0.280 | 5.0 7.3 | 1.12 1.5 | 0.022 0.032 | --- 0.12 |

1Applicable to diesel vehicles only

2Standards not applicable to diesel vehicles

3Option of certifying heavy-duty engines in vehicles up to 10,000 pounds GVWR using the light-duty truck (LDT) standards

Section 241.APPENDIX A   Emission Standards for Clean Fuel Vehicles

Section 241.TABLE B   Ultra-Low Emission Vehicle (ULEV) Standards for Light-Duty Clean Fuel Vehicles (g/mi)

| LIGHT-DUTY VEHICLE WEIGHT | POLLUTANT: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NMOG | CO | NOX | HCHO | PM1 |
| ALL LDV, LDT ≤ 6000 GVWR ≤ 3750 LVW 50,000 MILES 100,000 MILES | 0.040 0.055 | 1.7 2.1 | 0.2 0.3 | 0.008 0.011 | 0.08 0.04 |
| LDT ≤ 6000 GVWR > 3750 LVW ≤ 5750 LVW 50,000 MILES 100,000 MILES | 0.050 0.070 | 2.2 2.8 | 0.4 0.5 | 0.009 0.013 | 0.08 0.04 |
| LDT >6000 GVWR ≤ 3750 ALVW 50,000 MILES 100,000 MILES | 0.075 0.107 | 1.7 2.5 | 0.2 0.32 | 0.008 0.012 | --- 0.04 |
| LDT >6000 GVWR >3750 ALVW ≤ 5750 ALVW 50,000 MILES 120,000 MILES | 0.100 0.143 | 2.2 3.2 | 0.4 0.52 | 0.009 0.013 | --- 0.05 |
| LDT >6000 GVWR >5750 ALVW ≤ 8500 ALVW3 50,000 MILES 100,000 MILES | 0.117 0.167 | 2.5 3.7 | 0.6 0.82 | 0.011 0.016 | --- 0.06 |

1Applicable to diesel vehicles only

2Standards not applicable to diesel vehicles

3Option of certifying heavy-duty engines in vehicles up to 10,000 pounds GVWR using the light-duty truck (LDT) standards

Section 241.APPENDIX A   Emission Standards for Clean Fuel Vehicles

Section 241.TABLE C   NMOG Standards for Flexible-Fueled and Dual-Fueled Vehicles (g/mi)

|  |  | NMOG STANDARD1 |  |
| --- | --- | --- | --- |
| VEHICLE WEIGHT |  | 50,000 MILE | 100,000 MILE |
| ALL LDV, LDT, ≤ 6000 GVWR ≤ 3750 LVW |  | 0.075/0.125 | 0.09/0.156 |
| LDT | ≤ 6000 GVWR >3750LVW ≤ 5750 LVW | 0.100/0.160 | 0.130/0.200 |
| LDT | >6000 GVWR ≤ 3750 ALVW | 0.125/0.250 | 0.180/0.360 |
| LDT | >6000 GVWR >3750 ALVW ≤ 5750 ALVW | 0.160/0.320 | 0.230/0.460 |
| LDT | >6000 GVWR >5750 ALVW | 0.195/0.390 | 0.280/0.560 |

1The standards are presented for flexible-fueled and dual-fueled clean fuel vehicles when operating on clean alternative fuel and conventional fuel in the format "x/y" where x represents the NMOG standard when the vehicle is operated on a clean alternative fuel and y represents the NMOG standard when the vehicle is operated on a conventional fuel.

Section 241.APPENDIX A   Emission Standards for Clean Fuel Vehicles

Section 241.TABLE D   Emission Standards for Model Year 1998 and Later Heavy-Duty Vehicles (g/bhp-hr)

| VEHICLE TYPE | THC | NOX | NMHC + NOX | CO | PM1 | OMHCE | HCHO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GASOLINE ≤ 14,000 GVWR | 1.1 | 4.0 | --- | 14.4 | --- | 1.1 | --- |
| GASOLINE > 14,000 GVWR | 1.9 | 4.0 | --- | 37.1 | --- | 1.9 | --- |
| DIESEL | 1.3 | 4.0 | --- | 15.5 | 0.10 | 1.3 | --- |
| LEV CERTIFIED FUEL | 2 | 2 | 3.8 | 2 | 2 | 2 | --- |
| LEV CERTIFIED CALIF. FUEL | 2 | 2 | 3.5 | 2 | 2 | 2 | --- |
| ULEV | 2 | 2 | 2.5 | 7.2 | 0.05 | 2 | 0.025 |
| ILEV | 2 | 2 | 2.5 | 14.4 | 0.10 | 2 | 0.025 |

1Standards for particulate matter (PM) apply only to diesel-fueled vehicles.

2HD CFVs must meet conventional vehicle standards for THC, NOx, CO, PM, and OMHCE