Section 244.161   Advisory, Alert and Emergency Levels:

| Pollutant | Averaging Time | Advisory | Yellow Alert | Red Alert | Emergency |
| --- | --- | --- | --- | --- | --- |
| Sulfur dioxide (ppm) | 2-hour | 0.30 | -- | -- | -- |
|  | 4-hour | -- | 0.30 | 0.35 | 0.40 |
| (PM10 µg/m3 | 2-hour | 420 | -- | -- | -- |
|  | 24-hour | -- | 350 | 420 | 500 |
| Carbon Monoxide (ppm) | 2-hour | 30 | -- | -- |  |
|  | 8-hour | -- | 15. | 30. | 40. |
| Ozone (ppm) | 1-hour | 0.12 | 0.20 | 0.30 | 0.50 |
| Nitrogen dioxide (ppm) | 2-hour | 0.40 | -- | -- | -- |
|  | 1-hour | -- | 0.60 | 1.20 | 1.60 |
|  | 24-hour | -- | 0.15 | 0.30 | 0.40 |

(Source:  Amended at 16 Ill. Reg. 8191, effective May 15, 1992)