Section 276.507  On-Road Remote Sensing Test Systems Functional Requirements and Performance Criteria

a)	The on-road remote sensing test equipment shall consist of the following:

1)	An infrared SDM capable of measuring the concentration of HC, CO, and CO2 present in an in-use vehicle;

2)	A camera system that automatically photographs vehicle license plates;

3)	A system that links each emission record to the correct vehicle license plate photograph; and

4)	A laser based speed and acceleration measurement system.

b)	The Infrared SDM shall be of a design certified to meet the following accuracy requirements:

| Pollutant | Range | Accuracy |
| --- | --- | --- |
| HC | ALL | ±150 ppm or ±15 percent of expected HC Concentration |
| CO | ≤ 3.0 percent | ±10 percent or 0.25 percent (Whichever is greater) |
|  | > 3.0 percent | ±15 percent |

c)	The speed and acceleration measurement system shall be of a design certified to measure vehicle speed to within ±0.5 miles per hour and vehicle acceleration to within ±0.3 miles per hour per second at the moment exhaust is measured.

(Source:  Amended at 35 Ill. Reg. 11268, effective June 28, 2011)