Section 364.204  F2 Factor

F2 is a factor which evaluates the adequacy of existing facilities for treating the existing wastewater load to design levels.  It is calculated as follows:

| F2 | = | PE BOD, (existing wastewater load, industrial and domestic) |
| --- | --- | --- |
|  |  | PE BOD, existing design capacity |

| x | daily average flow, existing load | +1 |
| --- | --- | --- |
|  | daily average flow, existing design capacity |  |