Section 375.204  Phase III – Detailed Water Quality Evaluation

Areas to be addressed in a detailed water quality survey shall include, consistent with 35 Ill. Adm. Code 306.361(d):

a)	water column chemistry –

1)	upstream/downstream and overflow sampling;

2)	wet weather/dry weather sampling;

3)	diurnal sampling for dissolved oxygen (D.O.);

4)	parameters generally will include BOD5, suspended solids, volatile suspended solids, ammonia.  Other parameters may become important depending on the nature of the overflow; and

5)	other parameters as deemed appropriate relative to 35 Ill. Adm. Code 375.302(a)(8).

b)	sediment analysis –

1)	sediment oxygen demand;

2)	percent volatile matter;

3)	particle size distribution; and

4)	other parameters as deemed appropriate relative to 35 Ill. Adm. Code 375.302(a)(8).

c)	biological survey –

1)	benthic macroinvertebrates;

2)	fishes; and

3)	plant productivity assessment (algal blooms).

d)	hydraulic and hydrologic factors including storm hydrographs for receiving stream and overflow.