Section 570.APPENDIX A   Design Procedure Outlined

DESIGN PROCEDURE OUTLINED

1.	Collect Site Specific Data

a.	Types and Areas (sft) contributing drainage

b.	Slope of Field Application Area (FAA) – Appendix D

c.	Soil Infiltration Rate (SI) of FAA – Appendix C

2.	Calculate Runoff Volume and Total Drainage Area

Working Table in Appendix B

3.	Settling Basin Design

4.5 cft/100 sft of drainage area + 10% extra volume

Dimensions from Appendix I

4.	Field Application Area Design

FAA = (VR x 12)/((2 x SI) - 1.69) (square feet)

Dimensions from Appendix E

5.	Calculate Flow onto Field Application Area

Flows from Appendix G

or

Qf = (.0026) (FAA) (gpm)

6.	Effluent Transport System Design

Recommended Designs from Appendix F

7.	Junction Box Design

Recommended Design from Appendix H

8.	Distribution Manifold Design

Recommended Designs from Appendix H