Section 570.APPENDIX E   Tables to Determine Dimensions of Field Application Areas

|  |  | Tables to Determine Dimensions of Field Application Areas |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table 1: |  | How to Use Table 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  ENTER |  |  |  |  |  | Slope (%) |  |  |  |  |  Read Down |  |  |  |  |
|  |  |  |  |  | Length (FT) |  |  |  |  |  |  |  |  |  |  |
| WIDTH (FT) |  |  |  |  |  |  |  |  |  |  Field Application Area |  |  |  |  |  |
| 1. Enter at slope of field application area from Appendix D. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Read down column and find corresponding length of field application area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Continue down column stopping at area closest to that previously calculated for your site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Read left to find width of field application area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Table 2: |  |  | Runoff Field Application Areas (square feet) |  |  |  |  |  |  |  |  |  |  |  |  |
| Slope (%) Length (ft) |  |  | 0.5 300 | 0.7 375 |  |  | 1.0 425 |  |  | 1.5 525 | 2.0 600 |  | 3.0 750 | 4.0 800 |  |
| Width (ft) | 20 |  | 6,000 | 7,500 |  |  | 8,500 |  |  | 10,500 | 12,000 |  | 15,000 | 17,000 |  |
|  | 25 |  | 7,500 | 9,375 |  |  | 10,625 |  |  | 13,125 | 15,000 |  | 18,750 | 21,250 |  |
|  | 30 |  | 9,000 | 11,250 |  |  | 12,750 |  |  | 15,750 | 18,000 |  | 22,500 | 25,500 |  |
|  | 35 |  | 10,500 | 13,125 |  |  | 14,875 |  |  | 18,375 | 21,000 |  | 26,250 | 29,750 |  |
|  | 40 |  | 12,000 | 15,000 |  |  | 17,000 |  |  | 21,000 | 24,000 |  | 30,000 | 34,000 |  |
|  | 45 |  | 13,500 | 16,875 |  |  | 19,125 |  |  | 23,625 | 27,000 |  | 33,750 | 38,250 |  |
|  | 50 |  | 15,000 | 18,750 |  |  | 21,250 |  |  | 26,350 | 30,000 |  | 37,500 | 42,500 |  |
|  | 55 |  | 16,500 | 20,625 |  |  | 23,375 |  |  | 28,875 | 33,000 |  | 41,250 | 46,750 |  |
|  | 60 |  | 18,000 | 22,500 |  |  | 25,500 |  |  | 31,500 | 36,000 |  | 45,000 | 51,000 |  |
|  | 65 |  | 19,500 | 24,375 |  |  | 27,625 |  |  | 34,125 | 39,000 |  | 48,750 | 55,250 |  |
|  | 70 |  | 21,000 | 26,250 |  |  | 29,750 |  |  | 36,750 | 42,000 |  | 52,500 | 59,500 |  |
|  | 75 |  | 22,500 | 28,125 |  |  | 31,875 |  |  | 39,375 | 45,000 |  | 56,250 | 63,750 |  |
|  | 80 |  | 24,000 | 30,000 |  |  | 34,000 |  |  | 42,000 | 48,000 |  | 60,000 | 68,000 |  |
|  | 85 |  | 25,500 | 31,875 |  |  | 36,125 |  |  | 44,625 | 51,000 |  | 63,750 | 72,250 |  |
|  | 90 |  | 27,000 | 33,750 |  |  | 38,250 |  |  | 47,250 | 54,000 |  | 67,500 | 76,500 |  |
|  | 95 |  | 28,500 | 35,625 |  |  | 40,375 |  |  | 49,875 | 57,000 |  | 71,250 | 80,750 |  |
|  | 100 |  | 30,000 | 37,500 |  |  | 42,500 |  |  | 52,500 | 60,000 |  | 75,000 | 85,000 |  |