Section 602.240  Plans

a)	Unless otherwise specified by the Agency, plans submitted to the Agency must provide the following:

1)	a suitable title;

2)	the name of the owner of the community water supply;

3)	the area or institution to be served;

4)	a scale;

5)	a north point;

6)	the data used;

7)	the boundaries of the municipality or  area to be served;

8)	the date and the name and address of the designing engineer;

9)	a professional engineer's seal and signature to satisfy Section 602.105(a)(4);

10)	the locations and sizes of existing water mains;

11)	the locations and nature of existing water works structures and appurtenances affecting the proposed construction, noted on one sheet;

12)	the locations of any petroleum storage tanks within 400 feet of the proposed construction;

13)	dimensions, elevations and explanatory notes; and

14)	details as specified in Sections 620.245 through Section 602.260.

b)	Plans must be drawn to a scale that will describe the proposed structures and equipment.

c)	The size of plans submitted to the Agency must not exceed 24 inches by 36 inches.

(Source:  Added at 40 Ill. Reg. 6799, effective April 15, 2016)