Section 611.313  Maximum Residual Disinfectant Levels (MRDLs)

a)	MRDLs:

| Disinfectant residual | MRDL (mg/L) |
| --- | --- |
| Chlorine | 4.0 (as Cl2) |
| Chloramines | 4.0 (as Cl2) |
| Chlorine dioxide | 0.8 (as ClO2) |

b)	Compliance

1)	CWSs and NTNCWSs.  A Subpart B system must comply with this Section.

2)	Transient NCWSs.  A Subpart B system supplier must comply with the chlorine dioxide MRDL.

c)	USEPA identified the best technology, treatment techniques, or other means available for complying with the MRDLs in subsection (a):  controlling treatment processes to reduce disinfectant demand and controlling disinfection treatment processes to reduce disinfectant levels.

BOARD NOTE:  This Section derives from 40 CFR 141.65.

(Source:  Amended at 47 Ill. Reg. 16486, effective November 2, 2023)