Section 616.402  Prohibitions

a)	Under Sections 14.2(a), 14.2(c), and 14.3(e) of the Act, a person must not cause or allow the construction or operation of any landfill unit that is:

1)	Located wholly or partially within a minimum setback zone and that is either a new potential primary source or a new potential secondary source, except as specified in Sections 616.104; or

2)	Located wholly or partially within a maximum setback zone and that is a new potential primary source, except as specified in Section 616.104.

b)	A person must not cause or allow the disposal of special waste in a new on-site landfill unit within a regulated recharge area if the distance from the wellhead of the community water supply well to the landfill unit is 2500 feet or less, except as provided in Section 616.105.

(Source:  Amended at 47 Ill. Reg. 7631, effective May 16, 2023)