Section 620.305  Preventive Notification Procedures

a)	For groundwater quality monitoring under Section 620.302, a preventive notification must occur whenever a contaminant:

1)	Specified in Section 620.310(a)(3)(A) is detected (except due to natural causes) in Class I groundwater;

2)	Denoted as a carcinogen under Section 620.410(b) is detected in Class I groundwater; or

3)	Subject to a standard under Section 620.430 is detected (except due to natural causes) in Class III groundwater.

b)	When a preventive notification is required for groundwater that is monitored by a regulated entity for the subject contaminant, the owner or operator of the site must:

1)	Confirm the detection by resampling the monitoring well within 30 days after the date on which the first sample analyses are received; and

2)	Provide a preventive notification to the appropriate regulatory agency of the results of the resampling analysis within 30 days after the date on which the sample analyses are received, but no later than 90 days after the results of the first samples were received.

c)	When a preventive notification is required for groundwater that is monitored by a regulatory agency, the agency must notify the owner or operator of the site where the detection has occurred.  The owner or operator must:

1)	Confirm the detection by resampling within 30 days after the date of the notice by the regulatory agency; and

2)	Provide preventive notification to the regulatory agency of the results of the resampling analysis within 30 days after the date on which the sample analyses are received, but no later than 90 days after the results of the first samples were received.

d)	When a preventive notification of a confirmed detection has been provided by an owner or operator under this Section, additional detections of the same contaminant do not require further notice if the groundwater quality conditions are substantially unchanged or that preventive response is underway for the contaminant.

(Source:  Amended at 49 Ill. Reg. 4488, effective March 28, 2025)