Section 724.936  Reporting Requirements

a)	A semiannual report must be submitted by owners and operators subject to the requirements of this Subpart AA to the Agency by dates specified in the RCRA permit.  The report must include the following information:

1)	The USEPA identification number (35 Ill. Adm. Code 722.112), name, and address of the facility.

2)	For each month during the semiannual reporting period the following:

A)	Dates when the control device did the following:

i)	Exceeded or operated outside of the design specifications, as defined in Section 724.935(c)(4); and

ii)	Such exceedances were not corrected within 24 hours, or that a flare operated with visible emissions, as defined by Reference Method 22 monitoring;

B)	The duration and cause of each exceedance or visible emissions; and

C)	Any corrective measures taken.

b)	If during the semiannual reporting period, the control device does not exceed or operate outside of the design specifications, as defined in Section 724.935(c)(4), for more than 24 hours or a flare does not operate with visible emissions, as defined in Section 724.933(d), a report to the Agency is not required.

(Source:  Amended at 40 Ill. Reg. 11726, effective August 9, 2016)