Section 729.241  Aqueous Solutions of Halogenated Compounds

a)	No person shall cause or allow to be placed in any landfill any hazardous waste containing an aqueous liquid phase containing more than 14,000 mg of halogenated compounds in any 1 kg.

b)	An aqueous liquid phase containing more than 10,000 mg of total organic halogen, as measured by the TOX test, in any 1 kg is assumed to contain more than 14,000 mg of halogenated compounds per kilogram, unless the contrary is shown by a more definitive test whose validity is demonstrated to the Agency.