Section 742.APPENDIX C   Tier 2 Illustrations and Tables

Section 742.TABLE B   SSL Parameters

| Symbol | Parameter |  | Units | Source |  | Parameter Value(s) |
| --- | --- | --- | --- | --- | --- | --- |
| AT | Averaging Time for Noncarcinogens in Ingestion Equation |  | yr |  |  | Residential = 6 Industrial/Commercial = 25 Construction Worker = 0.115 |
| AT | Averaging Time for Noncarcinogens in Inhalation Equation |  | yr |  |  | Residential = 30 Industrial/Commercial = 25 Construction Worker = 0.115 |
| ATc | Averaging Time for Carcinogens |  | yr | SSL |  | 70 |
| BW | Body Weight |  | kg |  |  | Residential = 15, noncarcinogens 70, carcinogens Industrial/Commercial = 70 Construction Worker = 70 |
| Csat | Soil Saturation Concentration |  | mg/kg | Appendix A, Table A or Equation S29 in Appendix C, Table A |  | Chemical-Specific or Calculated Value |
| Cw | Target Soil Leachate Concentration |  | mg/L | Equation S18 in Appendix C, Table A |  | Groundwater Standard, Health Advisory concentration, or Calculated Value |
| d | Mixing Zone Depth |  | m | SSL or Equation S25 in Appendix C, Table A |  | 2 m or Calculated Value |
| da | Aquifer Thickness |  | m | Field Measurement |  | Site-Specific |
| ds | Depth of Source (Vertical thickness of contamination) |  | m | Field Measurement or Estimation |  | Site-Specific |
| Symbol | Parameter |  | Units | Source |  | Parameter Value(s) |
| DA | Apparent Diffusivity |  | cm2/s | Equation S10 in Appendix C, Table A |  | Calculated Value |
| Di | Diffusivity in Air |  | cm2/s | Appendix C, Table E |  | Chemical-Specific |
| Dw | Diffusivity in Water |  | cm2/s | Appendix C, Table E |  | Chemical-Specific |
| DF | Dilution Factor |  | unitless | Equation S22 in Appendix C, Table A |  | 20 or Calculated Value |
| ED | Exposure Duration for Ingestion of Carcinogens |  | yr |  |  | Industrial/Commercial = 25 Construction Worker = 1 |
| ED | Exposure Duration for Inhalation of Carcinogens |  | yr |  |  | Residential = 30 Industrial/Commercial = 25 Construction Worker = 1 |
| ED | Exposure Duration for Ingestion of Noncarcinogens |  | yr |  |  | Residential = 6 Industrial/Commercial = 25 Construction Worker = 1 |
| ED | Exposure Duration for Inhalation of Noncarcinogens |  | yr |  |  | Residential = 30 Industrial/Commercial = 25 Construction Worker = 1 |
| ED | Exposure Duration for the Direct Ingestion of Groundwater |  | yr |  |  | Residential = 30 Industrial/Commercial = 25 Construction Worker = 1 |
| EDM-L | Exposure Duration for Migration to Groundwater Mass-Limit Equation S28 |  | yr | SSL |  | 70 |
| EF | Exposure Frequency |  | d/yr |  |  | Residential = 350 Industrial/Commercial = 250 Construction Worker = 30 |
| F(x) | Function dependent on Um/Ut |  | unitless | SSL |  | 0.194 |
| foc | Organic Carbon Content of Soil |  | g/g | SSL or Field Measurement (See Appendix C, Table F) |  | Surface Soil = 0.006 Subsurface soil = 0.002, or Site-Specific |
| GWobj | Groundwater Remediation Remediation Objective |  | mg/L | Appendix B, Table E, 35 IAC 620.Subpart F, or Equation S23 in Appendix C, Table A |  | Chemical-Specific or Calculated |
| H' | Henry's Law Constant |  | unitless | Appendix C, Table E |  | Chemical-Specific |
| i |  | Hydraulic Gradient | m/m | Field Measurement (See Appendix C, Table F) |  | Site-Specific |
| I |  | Infiltration Rate | m/yr | SSL |  | 0.3 |
| IM-L |  | Infiltration Rate for Migration to Groundwater Mass-Limit Equation S28 | m/yr | SSL |  | 0.18 |
| IFsoil-adj (residential) |  | Age Adjusted Soil Ingestion Factor for Carcinogens | (mg-yr)/ (kg-d) | SSL |  | 114 |
| IRsoil |  | Soil Ingestion Rate | mg/d |  |  | Residential = 200 Industrial/Commercial = 50 Construction Worker = 480 |
| IRW |  | Daily Water Ingestion Rate | L/d |  |  | Residential = 2 Industrial/Commercial = 1 |
| K |  | Aquifer Hydraulic Conductivity | m/yr | Field Measurement (See Appendix C, Table F) |  | Site-Specific |
| Kd (Non-ionizing organics) |  | Soil-Water Partition Coefficient | cm3/g or L/kg | Equation S19 in Appendix C, Table A |  | Calculated Value |
| Kd (Ionizing organics) |  | Soil-Water Partition Coefficient | cm3/g or L/kg | Equation S19 in Appendix C, Table A |  | Chemical and pH-Specific (see Appendix C, Table I) |
| Kd (In-organics) |  | Soil-Water Partition Coefficient | cm3/g or L/kg | Appendix C, Table J |  | Chemical and pH-Specific |
| Koc |  | Organic Carbon Partition Coefficient | cm3/g or L/kg | Appendix C, Table E or Appendix C, Table I |  | Chemical-Specific |
| Ks |  | Saturated Hydraulic Conductivity | m/yr | Appendix C, Table K Appendix C, Illustration C |  | Site-Specific |
| L |  | Source Length Parallel to Groundwater Flow | m | Field Measurement |  | Site-Specific |
| PEF |  | Particulate Emission Factor | m3/kg | SSL or Equation S15 in Appendix C, Table A |  | Residential = 1.32 • 109 or Site-Specific Industrial/Commercial = 1.24 • 109 or Site-Specific |
| PEF′ |  | Particulate Emission Factor adjusted for Agitation (construction worker) | m3/kg | Equation S16 in Appendix C, Table A using PEF (industrial/commercial) |  | 1.24 • 108 or Site-Specific |
| Q/C (used in VF equations) |  | Inverse of the mean concentration at the center of a square source | (g/m2-s)/ (kg/m3) | Appendix C, Table H |  | Residential = 68.81 Industrial/Commercial = 85.81 Construction Worker = 85.81 |
| Q/C (used in PEF equations) |  | Inverse of the mean concentration at the center of a square source | (g/m2-s)/ (kg/m3) | SSL or Appendix C, Table H |  | Residential = 90.80 Industrial/Commercial = 85.81 Construction Worker = 85.81 |
| RfC |  | Inhalation Reference Concentration | mg/m3 | Illinois EPA: http://www.epa.state.il.us/land/taco/toxicity-values.xls |  | Toxicological-Specific (Note: for Construction Workers use subchronic reference concentrations) |
| RfDo |  | Oral Reference Dose | mg/(kg-d) | Illinois EPA: http://www.epa.state.il.us/land/taco/toxicity-values.xls |  | Toxicological-Specific (Note: for Construction Workers use subchronic reference doses) |
| ROsoil |  | Soil remediation objective | mg/kg | Equation S30 in Appendix C, Table A |  | Calculated value |
| ROsoil gas |  | Soil gas remediation objective | mg/m3 | Equation S30 in Appendix C, Table A |  | Calculated value |
| S |  | Solubility in Water | mg/L | Appendix C, Table E |  | Chemical-Specific |
| SFo |  | Oral Slope Factor | (mg/kg-d)-1 | Illinois EPA: http://www.epa.state.il.us/land/taco/toxicity-values.xls |  | Toxicological-Specific |
| T |  | Exposure Interval | s |  |  | Residential = 9.5 • 108 Industrial/Commercial = 7.9 • 108 Construction Worker = 3.6 • 106 |
| TM-L |  | Exposure Interval for Mass-Limit Volatilization Factor Equation S26 | yr | SSL |  | 30 |
| THQ |  | Target Hazard Quotient | unitless | SSL |  | 1 |
| TR |  | Target Cancer Risk | unitless |  |  | Residential = 10-6 at the point of human exposure Industrial/Commercial = 10-6 at the point of human exposure Construction Worker = 10-6 at the point of human exposure |
| Um |  | Mean Annual Windspeed | m/s | SSL |  | 4.69 |
| URF |  | Inhalation Unit Risk Factor | (μg/m3)-1 | Illinois EPA: http://www.epa.state.il.us/land/taco/toxicity-values.xls |  | Toxicological-Specific |
| Ut |  | Equivalent Threshold Value of Windspeed at 7 m | m/s | SSL |  | 11.32 |
| V |  | Fraction of Vegetative Cover | unitless | SSL or Field Measurement |  | 0.5 of Site-Specific |
| VF |  | Volatilization Factor | m3/kg | Equation S8 in Appendix C, Table A |  | Calculated Value |
| VF′ |  | Volatilization Factor adjusted for Agitation | m3/kg | Equation S9 in Appendix C, Table A |  | Calculated Value |
| VFM-L |  | Mass-Limit Volatilization Factor | m3/kg | Equation S26 in Appendix C, Table A |  | Calculated Value |
| VF′M-L |  | Mass-Limit Volatilization Factor adjusted for Agitation | m3/kg | Equation S27 in Appendix C, Table A |  | Calculated Value |
| η |  | Total Soil Porosity | Lpore/Lsoil | SSL or Equation S24 in Appendix C, Table A |  | 0.43, or Gravel = 0.25 Sand = 0.32 Silt = 0.40 Clay = 0.36, or Calculated Value |
| θa |  | Air-Filled Soil Porosity | Lair/Lsoil | SSL or Equation S21 in Appendix C, Table A | Surface Soil (top 1 meter) = 0.28 Subsurface Soil (below 1 meter) = 0.13, or Gravel = 0.05 Sand = 0.14 Silt = 0.24 Clay = 0.19, or Calculated Value |  |
| θw |  | Water-Filled Soil Porosity | Lwater/Lsoil | SSL or Equation S20 in Appendix C, Table A | Surface Soil (top 1 meter) = 0.15 Subsurface Soil (below 1 meter) = 0.30, or Gravel = 0.20 Sand = 0.18 Silt = 0.16 Clay = 0.17, or Calculated Value |  |
| ρb |  | Dry Soil Bulk Density | kg/L or g/cm3 | SSL or Field Measurement (See Appendix C, Table F) | 1.5, or Gravel = 2.0 Sand = 1.8 Silt = 1.6 Clay = 1.7, or Site-Specific |  |
| ρs |  | Soil Particle Density | g/cm3 | SSL or Field Measurement (See Appendix C, Table F) | 2.65, or Site-Specific |  |
| ρw |  | Water Density | g/cm3 | SSL | 1 |  |
| 1/(2b+3) |  | Exponential in Equation S20 | unitless | Appendix C, Table K Appendix C, Illustration C | Site-Specific |  |

(Source:  Amended at 37 Ill. Reg. 7506, effective May 15, 2013)