Section 742.APPENDIX C   Tier 2 Illustrations and Tables

Section 742.TABLE F   Methods for Determining Physical Soil Parameters

| Methods for Determining Physical Soil Parameters |  |  |
| --- | --- | --- |
| Parameter | Sampling Locationa | Method |
| b (soil bulk density) | Surface | ASTM-D 1556-90 Sand Cone Methodb |
|  |  | ASTM-D 2167-94 Rubber Balloon Methodb |
|  |  | ASTM-D 2922-91 Nuclear Methodb |
|  | Subsurface | ASTM-D 2937-94 Drive Cylinder Methodb |
| s (soil particle density) | Surface or Subsurface | ASTM-D 854-92 Specific Gravity of Soilb |
| w (moisture content) | Surface or Subsurface | ASTM-D 4959-89 (Reapproved 1994) Standardb |
|  |  | ASTM-D D 4643-93 Microwave Ovenb |
|  |  | ASTM-D D2216-92 Laboratory Determinationb |
|  |  | ASTM-D D3017-88 (Reapproved 1993) Nuclear Methodb |
|  |  | Equivalent USEPA Method (e.g., sample preparation procedures described in methods 3541 or 3550) |
| foc (fraction organic carbon content) | Surface or Subsurface | ASTM-D 2974-00 Moisture, Ash, and Organic Matter appropriately adjusted to estimate the fraction of organic carbon as stated in Nelson and Sommers (1982)b |
| or θT (total soil porosity) | Surface or Subsurface (calculated) | Equation S24 in Appendix C, Table A for SSL Model, or Equation R23 in Appendix C, Table C for RBCA Model, or Equation J&E16 in Appendix C, Table L for J&E Model |
| θa or θas (air-filled soil porosity) | Surface or Subsurface (calculated) | Equation S21 in Appendix C, Table A for SSL Model, or Equation R21 in Appendix C, Table C for RBCA Model, or Equation J&E18 in Appendix C, Table L for J&E Model |
| θw or θws (water-filled soil porosity) | Surface or Subsurface (calculated) | Equation S20 in Appendix C, Table A for SSL Model, or Equation R22 in Appendix C, Table C for RBCA Model, or Equation J&E17 in Appendix C, Table L for J&E Model |
| K (hydraulic conductivity) | Surface or Subsurface | ASTM-D 5084-90 Flexible Wall Permeameterb |
|  |  | Pump Test |
|  |  | Slug Test |
| i (hydraulic gradient) | Surface or Subsurface | Field Measurement |

a.	This is the location where the sample is collected

b	As incorporated by reference in Section 742.120.

(Source:  Amended at 37 Ill. Reg. 7506, effective May 15, 2013)