Section 811.207  Load Checking

a)	The operator shall not accept wastes for disposal at an inert waste landfill unless it is accompanied by documentation that such wastes are inert based on testing of the leachate from such wastes performed in accordance with the requirements of Section 811.202.

b)	The operator shall institute and conduct a random load checking program at each inert waste facility in accordance with the requirements of Section 811.323 except that this program shall also be designed:

1)	to detect and discourage attempts to dispose non-inert wastes at the landfill;

2)	to require the facility's inspector examine at least one random load of solid waste delivered to the landfill on a random day each week; and

3)	to require the operator to test one randomly selected waste sample in accordance with Section 811.202(a) and (b) to determine if the waste is inert.

b)	The operator shall include the results of the load checking in the Annual Report submitted to the Agency in accordance with 35 Ill. Adm. Code 813.501 for permitted facilities and 35 Ill. Adm. Code 815.Subpart C for non-permitted facilities.