Section 812.106  Site Location Map

All permit applications shall contain a site location map on the most recent United States Geological Survey (USGS) quadrangle of the area from the 7 1/2 minute series (topographic), or on such other map whose scale clearly shows the following information:

a)	The permit area and all adjacent property, extending at least 1000 meters (3300 feet) beyond the boundary of the facility;

b)	All surface waters;

c)	The prevailing wind direction;

d)	All rivers designated for protection under the Wild and Scenic Rivers Act (16 U.S.C. 1271 et seq.);

e)	The limits of all 100-year floodplains;

f)	All natural areas designated as a Dedicated Illinois Nature Preserve pursuant to the Illinois Natural Areas Preservation Act (Ill. Rev. Stat. 1989, ch. 105, par. 701 et seq.);

g)	All historic and archaeological sites designated by the National Historic Preservation Act (16 U.S.C. 470 et seq.) and the Illinois Historic Preservation Act (Ill. Rev. Stat. 1989 ch. 127, par. 133d1 et seq.);

h)	All areas identified as critical habitat pursuant to the Endangered Species Act (16 U.S.C. 1531 et seq.) and the Illinois Endangered Species Protection Act (Ill. Rev. Stat. 1989, ch. 8, par. 331 et seq.); and

i)	All main service corridors, transportation routes, and access roads to the facility.