Section 812.313  Design of the Final Cover System

The permit application shall contain documentation for the final cover system to demonstrate compliance with 35 Ill. Adm. Code 811.314, including:

a)	Material specifications;

b)	Placement techniques;

c)	Estimates of settling;

d)	A description of final protective cover, including a description of the soil and the depth necessary to maintain the proposed land use of the area;

e)	A description showing how the low permeability layer will tie into the liner system; and

f)	A construction quality assurance program, pursuant to 35 Ill. Adm. Code 811.Subpart E, which provides that the cover is constructed in compliance with all applicable requirements of 35 Ill. Adm. Code 811.