Section 812.317  Groundwater Monitoring Program

The permit application shall contain a groundwater monitoring plan which demonstrates compliance with 35 Ill. Adm. Code 811.318 and 811.319 and which includes the following information:

a)	A site plan map showing all zones of attenuation;

b)	Distance from the ground surface to the bottom of the uppermost aquifer;

c)	The location and depth of all groundwater monitoring points;

d)	The design of the groundwater monitoring wells, with a description of the materials to be used in constructing each well;

e)	A list of the parameters to be tested at each monitoring point;

f)	A concentration versus time profile for each monitoring point, showing the maximum allowable concentration at that monitoring point for the 100 years after the closure of the unit;

g)	A description of the sampling procedure to be followed;

h)	A description of the preservation techniques to be utilized;

i)	A description of the chain of custody, packing and transportation plans for all samples to meet the requirements of 35 Ill. Adm. Code 811.318(e);

j)	A description of the laboratory analysis, including laboratory procedures, quality control, and error detection;

k)	A description of the statistical analysis techniques to be used for evaluating the monitoring data;

l)	A description of the groundwater quality standards applicable at the facility pursuant to 35 Ill. Adm. Code 811.320, including a specific numerical value for each constituent and including an evaluation of the background concentrations of each constituent to be monitored; and

m)	A description of the statistical method to be utilized when evaluating groundwater data.