Section 845.620  Hydrogeologic Site Characterization

a)	The owner or operator of the CCR surface impoundment must design and implement a hydrogeologic site characterization.

b)	The hydrogeologic site characterization must include, but is not limited to, the following:

1)	Geologic well logs/boring logs;

2)	Climatic aspects of the site, including seasonal and temporal fluctuations in groundwater flow;

3)	Identification of nearby surface water bodies and drinking water intakes;

4)	Identification of nearby pumping wells and associated uses of the groundwater;

5)	Identification of nearby dedicated nature preserves;

6)	Geologic setting;

7)	Structural characteristics;

8)	Geologic cross-sections;

9)	Soil characteristics;

10)	Identification of confining layers;

11)	Identification of potential migration pathways;

12)	Groundwater quality data;

13)	Vertical and horizontal extent of the geologic layers to a minimum depth of 100 feet below land surface, including lithology and stratigraphy;

14)	A map displaying any known underground mines beneath a CCR surface impoundment;

15)	Chemical and physical properties of the geologic layers to a minimum depth of 100 feet below land surface;

16)	Hydraulic characteristics of the geologic layers identified as migration pathways and geologic layers that limit migration, including:

A)	Water table depth;

B)	Hydraulic conductivities;

C)	Effective and total porosities;

D)	Direction and velocity of groundwater flow; and

E)	Map of the potentiometric surface;

17)	Groundwater classification under 35 Ill. Adm. Code 620; and

18)	Any other information requested by the Agency that is relevant to the hydrogeologic site characterization.