Section 1100.304  Site Location Map

All permit applications must contain a site location map on the most recent United States Geological Survey (USGS) quadrangle of the area from the 7½ minute series (topographic) that clearly shows the following information:

a)	The site boundaries, the facility boundaries, and all adjacent property extending at least 1000 meters (3300 feet) beyond the facility boundaries;

b)	All surface waters;

c)	All potable water supply wells within 1000 meters (3300 feet) of the facility boundaries;

d)	All potable water supply well setback zones established pursuant to Section 14.2 or 14.3 of the Act;

e)	Any wellhead protection areas pursuant to Section 1428 of the Safe Drinking Water Act (SDWA) (42 USC 300f) and any sole source aquifer designated by the United States Environmental Protection Agency pursuant to Section 1424(e) of SDWA; and

f)	All main service corridors, transportation routes, and access roads to the site and facility.

(Source:  Amended at 36 Ill. Reg. 13892, effective August 27, 2012)