# Section 205.1760  Grounding

In areas used for patient care or treatment, all receptacles operating at over 100 volts shall be grounded by an insulated copper conductor, sized in accordance with Table 250-95 of NFPA 70 (see Section 205.115(a)(1)(B)), and installed with the branch conductors supplying these receptacles.

(Source:  Amended at 34 Ill. Reg. 7915, effective May 25, 2010)