Section 800.1400  Fish Cleaning Facilities

a)	There shall be no accumulation of fish offal on the ground in recreational areas.

b)	When fish cleaning facilities are provided in a recreational area, the facility must include the following:

1)	A watertight container with a tight-fitting lid for offal.

2)	A fish cleaning table resting on an impervious surface that is sloped to drain.

3)	A potable water hose bib fitted with a vacuum breaker.

c)	When fish cleaning facilities are provided within a recreational area, the fish cleaning table and impervious surface shall be cleaned after each day of use to control flies and insects.

(Source:  Amended at 10 Ill. Reg. 11076, effective July 1, 1986)