Section 855.425  Equipment Decontamination Enclosure System

The contractor shall provide or construct an equipment decontamination enclosure system consisting of two totally enclosed chambers as follows:

a)	The wash room or cleanup room shall be constructed with a curtained doorway to a designated area of the work area and a curtained doorway to the holding area.

b)	The holding area shall be constructed with a curtained doorway to the wash room and a lockable door to the exterior of the building.