Section 890.640  Prohibited Fixtures

a)	Drinking fountains shall not be installed in public toilet rooms.

b)	Fixed wooden, concrete, cement or tile wash trays or sinks used for food preparation, utensil washing, or hand washing shall not be installed in any food service establishment or commercial food establishment.

c)	Bathtub liners/inserts are prohibited unless all of the following conditions are met:

1)	Bathtub liners/inserts must be manufactured to an exact fit over existing bathtubs or be custom fabricated according to the dimensions of an existing bathtub;

2)	The floor (bottom surface) of the liner/insert must have a slip-resistant surface; and

3)	The bathtub liner/insert must be manufactured/fabricated from high-impact plexiglass/ABS or acrylic/plastic material complying with IAPMO/ANSI Z124.8 or from porcelain enameled formed steel complying with ASME A112.19.1/CSA 45.2.

(Source:  Amended at 38 Ill. Reg. 9940, effective April 24, 2014)