Section 905.APPENDIX A   Illustrations and Exhibits

Section 905.ILLUSTRATION C   List of Approved Plastic Pipe for Private Sewage

Disposal System

| TYPES OF PIPES |  | ASTM STANDARD | BUILDING SEWER 1 OR COMMON COLLECTOR |  | SEWER LINES 1 | ALL SUBSURFACE SEEPAGE SYSTEMS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 ft. from building to septic-aeration tank to 5 ft. beyond the septic tank, aeration tank or distribution box |  | Additional treatment facilities and sand filter distribution lines and collection lines |  |
| ABS | (DWV Schedule 40) | F628-06 | x |  | x | x |
| ABS | (DWV Schedule 40) | D2661-06 | x |  | x | x |
| ABS |  | D1527-99 (2005) | x |  | x | x |
| ABS | (Sewer Pipe) | D2751-05 | x2 |  | x2 | x2 |
| PVC |  | D1785-06 | x |  | x | x |
| PVC | (DWV Schedule 40) | D2665-07 | x |  | x | x |
| PVC | (DWV Schedule 40) | F891-04 | x |  | x | x |
| PVC | (Type PSM) (SDR 35) | D3034-06 | x2 |  | x2 | x2 |
| PVC | (Sewer & Drain PS-50) | F891-04 | x |  | x | x |
| PVC | (Sewer & Drain PS-25) | F891-04 | x |  | x |  |
| PVC | (Corrugated-Smoothwall) | F949-06 | x |  | x |  |
| PVC | (Std. or Perforated) | D2729-03 |  |  | x | x |
| PE | (Smoothwall) | R810-07 |  |  | x | x |
|  |  | AASHTO Standard M252-06 |  |  |  |  |
| PE | (Corrugated-Perforated) | F405-05 (Heavy Duty Only) |  |  |  | x |
| PE | (Corrugated-(Perforated) | F667-06 |  |  |  | x |

| x – | Indicates approved use. |  |
| --- | --- | --- |
| l Plastic materials shall not be co-mingled within this area except through the use of proper adapters. (See Illinois Plumbing Code.) When the building sewer is of a type of material that is different from the building drain, proper transition fittings shall be used. |  |  |
| 2 Pipe shall be SDR (Standard Dimension Ratio) 35 only. |  |  |
| Note: |  | The last 2 numbers of the ASTM Standard indicate the date of the edition. |

(Source:  Amended at 37 Ill. Reg. 14994, effective August 28, 2013)