Section 905.APPENDIX A   Illustrations and Exhibits

Section 905.ILLUSTRATION H   Subsurface Seepage System Size Determination

Section 905.EXHIBIT A   Gravel System

| Time (minutes) required for last 6 inches of water to fall |  | FOR RESIDENTIAL USE Required Absorption Area (sq ft)/bedroom) | FOR INSTITUTIONAL OR COMMERCIAL USE Allowable application rate (GPD/sq ft) (5) | Recommended depth from bottom of the trench to the limiting layer |
| --- | --- | --- | --- | --- |
| 18 - 60 |  | 200 | 1.0 |  |
| 90 |  | 210 | .95 | 3 feet |
| 120 |  | 235 | .85 |  |
| 150 |  | 265 | .75 |  |
| 180 |  | 290 | .69 |  |
| 240 |  | 320 | .62 |  |
| 300 |  | 350 | .57 | 2 feet |
| 360 |  | 385 | .52 |  |
| NOTE: |  |  |  |  |
| 1. | Absorption area is figured as trench bottom area in absorption trenches and bottom area in seepage beds. |  |  |  |
| 2. | Seepage beds require 1½ times the seepage field absorption area specified. |  |  |  |
| 3. | Over 360 is unsuitable for subsurface seepage systems. |  |  |  |
| 4. | Under 18 is unsuitable for subsurface seepage systems. |  |  |  |
| 5. | Divide the required total gallons per day by this number to get the number of square feet required |  |  |  |

(Source: Amended at 20 Ill. Reg. 2431, effective March 15, 1996)

Section 905.APPENDIX A   Illustrations and Exhibits

Section 905.ILLUSTRATION H   Subsurface Seepage System Size Determination

Section 905.EXHIBIT B   Gravelless System

| Time (minutes) required for last 6 inches of water to fall |  | FOR RESIDENTIAL USE Required Absorption Area (sq ft)/bedroom) |  | FOR INSTITUTIONAL OR COMMERCIAL USE Allowable application rate (GPD/sq ft) (5) |  | Recommended depth from bottom of the trench to the limiting layer |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 inch | 10 inch | 8 inch | 10 inch |  |
| 18 – 60 |  | 100 | 70 | 2.00 | 3.00 |  |
| 90 |  | 105 | 70 | 1.90 | 2.86 | 3 feet |
| 120 |  | 120 | 80 | 1.66 | 2.50 |  |
| 150 |  | 135 | 90 | 1.48 | 2.22 |  |
| 180 |  | 145 | 100 | 1.38 | 2.00 |  |
| 240 |  | 160 | 110 | 1.25 | 1.82 |  |
| 300 |  | 175 | 120 | 1.14 | 1.66 | 2 feet |
| 360 |  | 195 | 130 | 1.0 | 1.54 |  |
| NOTE: |  |  |  |  |  |  |
| 1. | Over 360 is unsuitable for subsurface seepage systems. |  |  |  |  |  |
| 2. | Under 18 is unsuitable for subsurface seepage systems. |  |  |  |  |  |
| 3. | Divide the required total gallons per day by this number to get the number of lineal feet required. |  |  |  |  |  |

(Source: Amended at 20 Ill. Reg. 2431, effective March 15, 1996)