Section 14.810 Application for Certificate of Approval

a)	New Heliports.  The Division will issue a Certificate of Approval for a heliport in accordance with Section 14.115, and, taking into consideration:

1)	the heliport’s proposed location;

2)	the heliport’s size and layout;

3)	the relationship of the proposed heliport to the then current State and Federal Airport and Airways System;

4)	whether there are safe areas available for expansion purposes;

5)	whether the adjoining areas are free from obstructions based on a

proper glide ratio;

6)	the nature of the terrain;

7)	the nature of the uses to which the proposed heliport will be put;

8)	the possibilities for future development; and

9)	the minimum standards contained in this Subpart H, including Section

14.Appendix G, Illustrations A, B and C and Section 14.Appendix H, Table A.  (See Section 48 of the Act.)

b)	Transfer of Certificate.  The Division will issue a new Certificate of Approval for the transfer of a heliport in accordance with Section 14.120(a).

c)	Modification of Certificate of Approval.  The Division will issue a new Certificate of Approval after completion of a heliport extension or alteration that requires a modification to the Certificate of Approval in accordance with Section 14.120(b).  For purposes of this Section the phrase, "extension or alteration" shall include the following:

1)	physical relocation of the FATO by more than 100' laterally or 25' vertically from the original certificated location;

2)	change in any approach/takeoff path by more than 30 degrees; or

3)	construction of one or more additional FATOs or TLOFs.  (See Section 47 of the Act.)