Section 14.APPENDIX H   Heliport/Vertiport Standards, Facility Requirements and Restrictions on Use

Section 14.TABLE B   Facility Requirements

| Item | Heliport/ Helistop Vertiport/ Vertistop Public-Use | Heliport/ Helistop Vertiport/ Vertistop Private-Use | Heliport/ Helistop Vertiport/ Vertistop Restricted Landing Area | Hospital Heliport/ Helistop Vertiport/ Vertistop |
| --- | --- | --- | --- | --- |
| Access Control a) Spectator b) Vehicular c) Perimeter | Required Required Required | Recommended Recommended Recommended | Recommended Recommended Recommended | Required Required Required Security and access may be controlled by hospital |
| Fire Protection | Required for heliports – one 20# extinguisher (two where fueling is present) | Required for heliports – one 20# extinguisher (two where fueling is present) | Recommended | Required – one 20# extinguisher (two where fueling is present) |
| First-Aid Kit | Required for heliport only | Recommended for heliport | Not Required | Not Required |
| Fuel & Oil Sales | Required for heliport during normal business hours Available by phone after business hours | Not Required | Not Required | Not Required |
| Hangar/Office | Required for heliport only | Not Required | Not Required | Not Required |
| Identification Beacons | Required for heliports | Recommended | Not Required | Recommended and required for all Trauma Centers |

| Item | Heliport/ Helistop Vertiport/ Vertistop Public-Use | Heliport/ Helistop Vertiport/ Vertistop Private-Use | Heliport/ Helistop Vertiport/ Vertistop Restricted Landing Area | Hospital Heliport/ Helistop Vertiport/ Vertistop |
| --- | --- | --- | --- | --- |
| Lead-in Lights and Arrows | Required for heliports | Recommended | Not Required | Recommended |
| Marked FATO and/or TLOF Identifiable from 500' AGL | Required | Required | Required | Required |
| Paved TLOF | Recommended | Recommended | Not Required | Recommended |
| Perimeter/Flood Lighting | Required for night use | Required for night use | Recommended for night use | Required for night use |
| Potable Water | Required for heliport during normal business hours Available by phone after business hours | Not Required | Not Required | Not Required |
| Horizontal Safety Fence for Heliports Elevated 30" or Higher | Required | Required | Required | Required |
| Sanitary Restroom | Required for heliport during normal business hours Available by phone after business hours | Not Required | Not Required | Not Required |
| Wind Direction / Velocity Indicator (must be lighted for night use) | Required | Required | Required | Required |

(Source:  Amended at 37 Ill. Reg. 15127, effective August 30, 2013)