Section 550.ILLUSTRATION B   Table of Entrance Layout Requirements

|  | NONCOMMERCIAL RURAL | NONCOMMERCIAL URBAN | COMMERCIAL RURAL |  | COMMERCIAL URBAN | INDUSTRIAL-COMMERCIAL-RECREATIONAL HGH-VOLUME TRAFFIC GENERATORS |
| --- | --- | --- | --- | --- | --- | --- |
| WIDTH OF DRIVE | 12' Min. 24' Max. |  | 35' Max. (60' Max. 6' from Edge of Pavement) |  | 35' Max. (85' Max. at Curb) | 2@24' or 35' Max. |
| RADII OF FLARE | 10' Min. 30' Max. | 5' Min. 15' Max. | 10' Min. 40' Max. |  | 10' Min. 25' Max. | 30 – 50' or 3 C.C. |
| ANGLE OF DRIVE | 60º - 90º | *45º - 90º |  |  |  |  |
| ISLAND AREA | ---- |  | 10' Min. at ROW 5' Min. Radius 10' Min 6' from Pavement |  | 6' Min. at Edge of Pavement and at ROW 5' Min. Radius | 4' – 18" Wide Median |
| DISTANCE FROM PROPERTY LINE (To any part of driveway or flare) | 0' Min. |  | 5' Min. (6' from Edge of Pavement) |  | 3' Min. | 10' Min. |
| DISTANCE FROM INTERSECTING STREET | **50' Min. | ***5' Min. | **50' Min. |  | ---- | **100' Min. |
|  | 5' Min. from beginning of flare to extension of intersecting road ROW |  |  |  |  |  |
| DISTANCE BETWEEN DRIVES | ---- |  |  | 10' Min. (6' from Edge of Pavement and at ROW) | 6' Min. (at Edge of Pavement and at ROW) | 440' Min. 660' Desirable |

*	45º permitted only for one-way drives; 60º minimum for two-way drives.

**	From edge of intersecting road to beginning of driveway flare.

***	Undisturbed length of curb between driveway flare and intersecting street flare.